# CLEANAIR ENGINEERING

GUIDE TO

HAZARDOUS SHIPPING

### Table of Contents

## Page

- CleanAir Hazardous Shipping Guide

- Limited Quantity Shipments and Non-Hazardous Reagents

- 4G Box/Can Packing Guide

- Acetone

- Air, Compressed (Zero Air)

- Buret Solution (Bisulphates, Aqueous Solution)

- Compressed Gas, N.O.S. (Air)

- Compressed Gas, N.O.S. (Nitrogen)

- Corrosive Liquid, Oxidizing, N.O.S. (5% Nitric Acid, 10% Hydrogen Peroxide)

- Disorb (Potassium Hydroxide Solution)

- Dichloromethane

- Dry Ice

- Ethanol

- Flammable Liquid, N.O.S. (50% Acetone, 50% Dichloromethane)

- Helium, Compressed

- Hexanes

- Hydrochloric Acid

- Hydrogen, Compressed

- Hydrogen Peroxide, Aqueous Solution (8% - 19%)

- Hydrogen Peroxide, Aqueous Solution (20% - 40%)

- Isopropanol

- Methanol

- Nitrogen, Compressed (Zero Nitrogen)

- Nitric Acid (Less than 20%)

- Nitric Acid (20% - 70%)

- Oxidizing Liquid, Corrosive, N.O.S. (4% Potassium Permanganate, 10% Sulphuric Acid)

- Oxsorb (Hydrochloric Acid)

- Potassium Hydroxide, Solid

- Potassium Permanganate

- Sodium Hydroxide, Solid

- Sulphuric Acid (Less than 51%)

- Sulphuric Acid (More than 51%)

- Toluene

### CLEANAIR HAZARDOUS SHIPPING GUIDE

The information on the following pages is intended to be used as a guide for shipping samples and chemicals by air using Federal Express as a carrier.

#### Hazardous Substances which can be Shipped as Water

Many substances, which would be considered hazardous at full strength, CleanAir uses as dilute solutions.  These can be considered non-hazardous and can be shipped as a regular package.

The following is a list of chemical reagents that are not considered hazardous at the concentrations CleanAir commonly uses.

Reagent					Method

3% Hydrogen Peroxide			Methods 6, 8

0.1 Normal Nitric Acid			Methods 12, 29

0.1 Normal Potassium Hydroxide		BIF Method 0013

0.l Normal Sodium Hydroxide		Method 26A, CARB 425

0.1 Normal Sulfuric Acid			Method 26

These reagents can only be shipped non-hazardous in low concentrations.  Full strength reagents and strong (concentrated) solutions need to be packed properly, and be accompanied by the required Dangerous Goods paperwork.

##### Important Phone Numbers

###### FEDERAL EXPRESS DANGEROUS GOODS HOTLIINE

1-800-238-5355

CHEMTEL (Emergency Phone Number)

1-800-255-3924

LIMITED QUANTITY SHIPMENTS

1.	Limited Quantity Shipments do not require a 4G box.

2.	The inner packaging of the shipping container remains the same.

3.	The packing instructions will have the letter “Y” followed by the

packing instruction number (ex. Y305).

4.	The words LIMITED QUANTITY or LTD. QTY. must appear under the

authorization column on the Hazardous Goods Declaration and on the

shipping box, underneath the UN number.

SHIPPING MORE THAN ONE HAZARDOUS SUBSTANCE

IN A SINGLE PACKAGE

1.	The substances must not react with each other (ex. strong acids and bases).

2.	The substances should not require segregation according to table 9.3A in the IATA Dangerous Goods Regulation book.

3.	The inner packaging of each item must comply with the relevant part of the packing instructions applicable to that item.

4.	The quantities of hazardous substances contained within the outer package must have a “Q” value less than 1.

A “Q” value is calculated by the following equation:

Q = n1/m1 + n2/m2 + n3/m3 . . .

Where:    	n  = The net amount of the substance being shipped

m = The maximum net quantity for that substance’s

packing instruction.