Correct Packing Method for Flammable/Organic Samples Shipped in a Small 4G Box

# Flammable Liquids in DOT Class 3 and Toxic Liquids in DOT Class 6.1

- No can is required for shipments of

Flammable/Group III Toxic liquids.

- All sample container lids must be sealed.

- All samples must be in a sealed plastic bag.

- A minimum of 1.5” of vermiculite should

surround every sample.

## Note:  Federal Express will only accept “Toxic” samples

in Packing Group III.  “Toxic” samples in Group I and II

will not be accepted.

### Maximum Capacity for Small 4G Box Shipments

- 8 – 250 ml or 500 ml glass or plastic

sample containers.

This shipping method has been tested for CleanAir and should be followed at all times to avoid accidental spills and broken sample containers.

Sample Containers in Plastic Bags

Vermiculite

Cardboard

Separation

Pad

Cardboard Inserts

Correct Packing Method for Flammable/Organic Samples Shipped in a Large 4G Box

# Flammable Liquids in DOT Class 3 and Toxic Liquids in DOT Class 6.1

Sample Containers in Plastic Bags

- No can is required for shipments of

Flammable/Group III Toxic liquids.

- All sample container lids must be sealed.

- All samples must be in a sealed plastic bag.

- A minimum of 1.5” of vermiculite should

surround every sample.

## Note:  Federal Express will only accept “Toxic” samples

in Packing Group III.  “Toxic” samples in Group I and II

will not be accepted.

### Maximum Capacity for Large 4G Box Shipments

- 18 – 250 ml or 500 ml glass or plastic

sample containers.

This shipping method has been tested for CleanAir and should be followed at all times to avoid accidental spills and broken sample containers.

Vermiculite

Cardboard Separation

Pad

Cardboard Inserts

Correct Packing Method for Corrosive/Oxidizer Samples Shipped

in a Metal Can Inside a Small 4G Box

Corrosive Liquids in DOT Class 8 and Oxidizing Liquids in DOT Class 5.1

Sample Containers in Plastic Bags

- A can is required for shipments of

Corrosive/Oxidizing Liquids.

- All sample container lids must be sealed.

- All samples must be in a sealed plastic bag.

- A minimum of 1.5” of vermiculite should

surround every sample.

### Maximum Capacity for Shipments

- 8 – 250 ml or 500 ml glass or plastic

sample containers.

This shipping method has been tested for CleanAir and should be followed at all times to avoid accidental spills and broken sample containers.

Vermiculite

Cardboard

Separation

Pad

Cardboard Inserts

Metal Shipping Can

Small 4G  Box

Correct Packing Method for Corrosive/Oxidizer Samples Shipped

in a Metal Can Inside a Large 4G Box

Corrosive Liquids in DOT Class 8 and Oxidizing Liquids in DOT Class 5.1

Sample Containers in Plastic Bags

Vermiculite

Cardboard Inserts

Metal Shipping Can

- A can is required for all shipments of

Corrosive/Oxidizing liquids.

- All sample container lids must be sealed.

- All samples must be in a sealed plastic bag.

- A minimum of 1.5” of vermiculite should

surround every sample.

### Maximum Capacity for Shipments

- 8 – 250 ml or 500 ml, 950  (1000) ml glass or plastic sample containers.

This shipping method has been tested for CleanAir and should be followed at all times to avoid accidental spills and broken sample containers.

Cardboard

Separation

Pad

Large 4G Box

Methods 5, 17, 23/0010, 201A, General

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Acetone | 3 | UN 1090 | II |  | Max Per Pkg 1 Liter or | Y305 | LTD. QTY. | For limited quantity shipments. |
|  |  |  |  |  | Max Per Pkg 5 Liters | 305 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 307 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

ACETONE

UN 1090

“Flammable Liquid" Label

Compressed Gases

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Air, Compressed | 2.2 | UN 1002 |  |  |  |  |  |  |
|  |  |  |  |  | 75 kg | 200 |  |  |
|  |  |  |  |  | or 150 kg | 200 |  | Requires a Cargo Aircraft Only Label |

SHIPPING WITHOUT A BOX

SHIPPING IN A BOX

Tips for Shipping Compressed Gas Cylinders:

- Proper shipping labels must appear on the outside of the cylinder (box).

- A shipping label with full address should be placed on the cylinder (box).

- Proper shipping name and UN number should be printed on the same side of the cylinder (box) as the hazard label.

- The valve cover must be securely in place before shipping.

- 	LABELS SHOULD NEVER OVERLAP!

“Non-flammable Gas” Label

This should be a

preprinted label.

Air,

Compressed

UN 1002

Air, Compressed

UN 1002

Orsat Chemicals – Buret Solutions

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Bisulphates, Aqueous Solution | 8 | UN 2837 | III |  | Max Per Pkg 1 Liter or | Y819 | LTD. QTY. | For limited quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 Liters | 819 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 821 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

BISULPHATES,

AQUEOUS

SOLUTION

UN 2837

UN 2837

“Corrosive” Label

Compressed Gases – Calibration Gases in Air

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Compressed Gas, N.O.S. (Air) | 2.2 | UN 1956 |  |  |  |  |  |  |
|  |  |  |  |  | 75 kg | 200 |  |  |
|  |  |  |  |  | or 150 kg | 200 |  | Requires a Cargo Aircraft Only Label |

SHIPPING WITHOUT A BOX

Tips for Shipping Compressed Gas Cylinders:

- Proper shipping labels must appear on the outside of the cylinder (box).

- A shipping label with full address should be placed on the cylinder (box).

- Proper shipping name and UN number should be printed on the same side of the cylinder (box) as the hazard label.

- The valve cover must be securely in place before shipping.

-     LABELS SHOULD NEVER OVERLAP!

SHIPPING IN A BOX

“Non-flammable Gas” Label

This should be a

preprinted label.

Compressed

Gas, N.O.S.

(AIR)

UN 1956

UN 1002

Compressed

Gas, N.O.S.

(AIR)

UN 1956

UN 1002

Compressed Gases – Calibration Gases in Nitrogen

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Compressed Gas, N.O.S. (Nitrogen) | 2.2 | UN 1956 |  |  |  |  |  |  |
|  |  |  |  |  | 75 kg | 200 |  |  |
|  |  |  |  |  | or 150 kg | 200 |  | Requires a Cargo Aircraft Only Label |

SHIPPING WITHOUT A BOX

SHIPPING IN A BOX

Tips for Shipping Compressed Gas Cylinders:

- Proper shipping labels must appear on the outside of the cylinder (box).

- A shipping label with full address should be placed on the cylinder (box).

- Proper shipping name and UN number should be printed on the same side of the cylinder (box) as the hazard label.

- The valve cover must be securely in place before shipping.

-     LABELS SHOULD NEVER OVERLAP!

“Non-flammable Gas” Label

This should be a

preprinted label.

Compressed

Gas, N.O.S.

(NITROGEN)

UN 1956

UN 1002

Compressed

Gas, N.O.S.

(NITROGEN)

UN 1956

UN 1002

Method 29 – Multi Metals Absorbing Solution

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Corrosive Liquid, Oxidizing, N.O.S. (5% Nitric Acid, 10% Hydrogen Peroxide) | 8 | UN 3093 | II | 5.1 | Max Per Pkg 0.5 Liter or | Y809 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 1 Liter | 809 |  |  |
|  |  |  |  |  | or Max Per Pkg 30 Liters | 813 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

Corrosive Liquid,

Oxidizing, N.O.S.

(5% Nitric Acid,

10% Hydrogen

Peroxide)

UN 3093

UN 2837

“Corrosive” Label

“Cargo Aircraft Only” Label

(if required)

“Oxidizer” Label

ORSAT Chemicals - Disorb

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Potassium Hydroxide Solution | 8 | UN 1814 | III |  | Max Per Pkg 1 Liter or | Y819 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 Liters | 819 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 821 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

POTASSIUM

HYDROXIDE

SOLUTION

UN1814

“Corrosive” Label

Methods 23/0010, 202

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Dichloromethane | 6.1 | UN 1593 | III |  | Max Per Pkg 2 Liters or | Y605 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 50 Liters | 605 |  |  |
|  |  |  |  |  | or Max Per Pkg 220 Liters | 612 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

DICHLOROMETHANE

UN1593

“Toxic 6.1” Label

General – Dry Ice

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Dry Ice | 9 | UN 1845 | III |  |  |  |  |  |
|  |  |  |  |  | Max Per Pkg 200 kg | 904 |  |  |
|  |  |  |  |  | or Max Per Pkg 200 kg | 904 |  | Requires a Cargo Aircraft Only Label |

Hazardous Markings (Example)

Tips for Shipping Dry Ice:

For non-hazardous substances being shipped on Dry Ice:

- The shipping container requires the markings “Dry Ice, UN1845” and the net weight in kilograms of the dry ice in the container.

- A shipper’s declaration is not required, but the section under Special Handling on the FedEx form concerning Dry Ice must be filled out.

For hazardous substances being shipped on Dry Ice:

- The information for Dry Ice must be included on the Shipper’s Declaration in addition to the information concerning the hazardous substance.

- The shipping container must have the appropriate markings for Dry Ice in addition to the markings for the other hazards.

- All packaging must be designed and constructed to allow the release of carbon dioxide gas to prevent a pressure buildup which could rupture the packaging.

-     LABELS SHOULD NEVER OVERLAP!

Required Only if Shipping Hazardous Substances on Dry Ice

THIS END UP

HAZARDOUS

SUBSTANCE

UN 0000

DRY ICE

UN 1845

2 kg

Hazard Label if Required

Class 9 Miscellaneous Label

General

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Ethanol | 3 | UN 1170 | II |  | Max Per Pkg 1 Liter or | Y305 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 Liters | 305 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 307 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

ETHANOL

UN 1170

“Flammable Liquid” Label

Method 23/0010

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Flammable Liquid, N.O.S. (50% Acetone, 50% Dichloromethane) | 3 | UN 1993 | II |  | Max Per Pkg 1 Liter or | Y305 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 Liters | 305 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 307 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

FLAMMABLE

LIQUID, N.O.S.

(50% Acetone,

50% Dichloromethane)

UN 1993

“Flammable Liquid” Label

Compressed Gases

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Helium, Compressed | 2.2 | UN 1046 |  |  |  |  |  |  |
|  |  |  |  |  | 75 kg | 200 |  |  |
|  |  |  |  |  | or 150 kg | 200 |  | Requires a Cargo Aircraft Only Label |

SHIPPING WITHOUT A BOX

SHIPPING IN A BOX

Tips for Shipping Compressed Gas Cylinders:

- Proper shipping labels must appear on the outside of the cylinder (box).

- A shipping label with full address should be placed on the cylinder (box).

- Proper shipping name and UN number should be printed on the same side of the cylinder (box) as the hazard label.

- The valve cover must be securely in place before shipping.

- 	LABELS SHOULD NEVER OVERLAP!

“Non-flammable Gas” Label

This should be a

preprinted label.

Helium,

Compressed

UN 1046

Helium, Compressed

UN 1046

General

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Hexanes | 3 | UN 1208 | II |  | Max Per Pkg 1 Liter or | Y305 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 Liters | 305 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 307 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

HEXANES

UN 1208

“Flammable Liquid” Label

Method 101A (29)/DNPH/General – Strong Hydrochloric Acid Solutions

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Hydrochloric Acid | 8 | UN 1789 | II |  | Max Per Pkg 0.5 Liter or | Y809 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 1 Liter | 809 |  |  |
|  |  |  |  |  | or Max Per Pkg 30 Liter | 813 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

HYDROCHLORIC

ACID

UN 1789

“Corrosive” Label

Compressed Gases

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Hydrogen, Compressed | 2.1 | UN 1049 |  |  |  |  |  | Not allowed on passenger aircraft. |
|  |  |  |  |  | 150 kg | 200 |  | Requires a Cargo Aircraft Only Label |

SHIPPING WITHOUT A BOX

SHIPPING IN A BOX

Tips for Shipping Compressed Gas Cylinders:

- Proper shipping labels must appear on the outside of the cylinder (box).

- A shipping label with full address should be placed on the cylinder (box).

- Proper shipping name and UN number should be printed on the same side of the cylinder (box) as the hazard label.

- A “Cargo Aircraft Only” label is required and should be placed under the shipping name and UN number.

- The valve cover must be securely in place before shipping.

- 	LABELS SHOULD NEVER OVERLAP!

“Flammable Gas” Label

This should be a

preprinted label.

Hydrogen,

Compressed

UN 1049

Hydrogen, Compressed

UN 1049

“Cargo Aircraft Only”        Label

General – Hydrogen Peroxide Solutions with more than 8%, but less than 20% Hydrogen Peroxide

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Hydrogen Peroxide, Aqueous Solution | 5.1 | UN 2984 | III |  | Max Per Pkg 1 Liter or | Y514 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 2.5 Liters | 514 |  |  |
|  |  |  |  |  | or Max Per Pkg 30 Liters | 515 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

HYDROGEN

PEROXIDE,

AQUEOUS SOLUTION

UN 2984

“Oxidizer” Label

Methods 6, 8, 29 – Hydrogen Peroxide Solutions with at least 20%, but less than 40% Hydrogen Peroxide

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Hydrogen Peroxide, Aqueous Solution | 5.1 | UN 2014 | II | 8 | Max Per Pkg 0.5 Liter or | Y501 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 1 Liter | 501 |  |  |
|  |  |  |  |  | or Max Per Pkg 5 Liters | 506 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

HYDROGEN

PEROXIDE,

AQUEOUS SOLUTION

UN 2014

“Corrosive” Label

“Oxidizer” Label

Methods 6, 8, General

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Isopropanol | 3 | UN 1219 | II |  | Max Per Pkg 1 Liter or | Y305 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 Liters | 305 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 307 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

ISOPROPANOL

UN 1219

“Flammable Liquid” Label

General

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Methanol | 3 | UN 1230 | II | 6.1 | Max Per Pkg 1 Liter or | Y305 | LTD. QTY. CA9301014 | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 1 Liter | 305 | CA9301014 |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 307 | CA9301014 | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

METHANOL

UN 1230

“Flammable Liquid” Label

Compressed Gases

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Nitrogen, Compressed | 2.2 | UN 1066 |  |  |  |  |  |  |
|  |  |  |  |  | 75 kg | 200 |  |  |
|  |  |  |  |  | or 150 kg | 200 |  | Required a Cargo Aircraft Only Label |

SHIPPING WITHOUT A BOX

SHIPPING IN A BOX

Tips for Shipping Compressed Gas Cylinders:

- Proper shipping labels must appear on the outside of the cylinder (box).

- A shipping label with full address should be placed on the cylinder (box).

- Proper shipping name and UN number should be printed on the same side of the cylinder (box) as the hazard label.

- The valve cover must be securely in place before shipping.

- 	LABELS SHOULD NEVER OVERLAP!

“Non-Flammable Gas” Label

This should be a

preprinted label.

Nitrogen,

Compressed

UN 1066

Nitrogen, Compressed

UN 1066

General – Nitric Acid Solutions other than red fuming, with less than 20% Nitric Acid

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Nitric Acid, 10% Solution | 8 | UN 2031 | II |  | Max Per Pkg 0.5 Liter or | Y807 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 1 Liter | 807 |  |  |
|  |  |  |  |  | or Max Per Pkg 30 Liters | 813 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

NITRIC ACID,

10% SOLUTION

UN 2031

“Corrosive” Label

General – Nitric Acid Solutions other than red fuming, with less than 70%, but more than 20% Nitric Acid

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Nitric Acid, 35% Solution | 8 | UN 2031 | II |  |  |  |  | Not Allowed on Passenger Aircraft |
|  |  |  |  |  | Max Per Pkg 30 Liters | 813 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

NITRIC ACID,

35% SOLUTION

UN 2031

“Corrosive” Label

“Cargo Aircraft Only” Label (required)

ORSAT Chemicals - Oxsorb

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Hydrochloric Acid | 8 | UN 1789 | III |  | Max Per Pkg 1 Liter or | Y819 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 Liters | 819 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 821 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

HYDROCHLORIC

ACID

UN 1789

“Corrosive” Label

Method 101A (29) – Mercury Absorbing Solution

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Oxidizing Liquid, Corrosive, N.O.S. (4% Potassium, Permanganate, 10% Sulphuric Acid) | 5.1 | UN 3098 | II | 8 | Max Per Pkg 0.5 Liter or | Y501 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 1 Liter | 501 |  |  |
|  |  |  |  |  | or Max Per Pkg 5 Liters | 506 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

OXIDIZING LIQUID,

CORROSIVE, N.O.S.

(4% Potassium,

Permanganate,

10% Sulphuric Acid)

UN 3098

“Corrosive” Label

“Cargo Aircraft Only” Label (if required)

“Oxidizer” Label

BIF Method 0013/General – Potassium Hydroxide Pellets

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Potassium Hydroxide, Solid | 8 | UN 1813 | II |  | Max Per Pkg 5 kg or | Y814 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 15 kg | 814 |  |  |
|  |  |  |  |  | or Max Per Pkg 50 kg | 816 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

POTASSIUM

HYDROXIDE, SOLID

UN 1813

“Corrosive” Label

Method 101A (29) – Potassium Permanganate (Solid)

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Potassium Permanganate | 5.1 | UN 1490 | II |  | Max Per Pkg 2.5 kg or | Y508 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 kg | 508 |  |  |
|  |  |  |  |  | or Max Per Pkg 25 kg | 511 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

POTASSIUM

PERMANGANATE

UN 1490

“Oxidizer” Label

General – Sodium Hydroxide Pellets

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Sodium Hydroxide, Solid | 8 | UN 1823 | II |  | Max Per Pkg 5 kg or | Y814 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 15 kg | 814 |  |  |
|  |  |  |  |  | or Max Per Pkg 50 kg | 816 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

SODIUM

HYDROXIDE,

SOLID

UN 1823

“Corrosive” Label

General – Sulphuric Acid Solutions with not more than 51% acid

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Sulphuric Acid | 8 | UN 2796 | II |  | Max Per Pkg 0.5 Liter or | Y809 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 1 Liter | 809 |  |  |
|  |  |  |  |  | or Max Per Pkg 30 Liters | 813 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

SULPHURIC

ACID

UN 2796

“Corrosive” Label

General – Sulphuric Acid Solutions with more than 51% acid

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Sulphuric Acid | 8 | UN 1830 | II |  | Max Per Pkg 0.5 Liter or | Y809 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 1 Liter | 809 |  |  |
|  |  |  |  |  | or Max Per Pkg 30 Liters | 813 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

SUPHURIC

ACID

UN 1830

“Corrosive” Label

Method 23

| Dangerous Goods Identifications |  |  |  |  | Quantity and Type Of Packing | Packing Inst. | Authorization | Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proper Shipping Name | Class or Division | UN or ID No. | Packing Group | Subsi- diary Risk |  |  |  |  |
| Toluene | 3 | UN 1294 | II |  | Max Per Pkg 1 Liter or | Y305 | LTD. QTY. | For Limited Quantity Shipments |
|  |  |  |  |  | Max Per Pkg 5 Liters | 305 |  |  |
|  |  |  |  |  | or Max Per Pkg 60 Liters | 307 |  | Requires a Cargo Aircraft Only Label |

Tips for Labeling the Shipping Container:

- The words “THIS END UP” should be clearly visible on the top panels of the box.

- A Shipping label with full address should be placed on the top panel of the box.

- Proper shipping name and UN number should be printed on the same side of the box as the preprinted up arrows.

- The proper shipping name and UN number should be printed clearly and legibly.  All printing should be about an inch tall.

- All appropriate hazard and handling labels should appear on the same side of the box as he up arrows and printing.

- The emergency phone number should be printed on an open side panel.

- If required, the “CARGO AIRCRAFT ONLY” label should be on the same side of the box as the hazard label, in the lower right hand corner.

-     LABELS SHOULD NEVER OVERLAP!

THIS END UP

TOLUENE

UN 1294

“Flammable Liquid” Label