|  | Fall Protection Saf-T-Grab Inspection Form |
| --- | --- |

Inspector Name:____________________________________________________ Date:__________________

Team Leader:_______________________________________________ Mfg. Date:____________________

Serial No:__________________Make:________________________________Model:___________________

To maintain their service life and high performance, all North Saf-T-Grab climbing sleeves should be formally inspected at least annually by a competent person.  Visual inspection should be performed before each use.  If any of the conditions listed below are found the equipment should be replaced before being used.

SAF-T-GRAB INSPECTION

PASS	 FAIL

Check that the two return springs on the side legs are in good working order and that the

springs return the sleeve to the lock position when the sleeve is parallel to the ground.

Inspect the entire sleeve for cracks, bends, and deformities or excessive wear.

Ensure all fasteners are secure and corrosion free.

Ensure the energy absorber has its protective clear cover and is intact and damage free.

Ensure the energy absorber has not been deployed and there are no signs of cuts, burns,

UV degradation or any other deformities.  Ensure all the stitching is still intact and that the wear pads do not show any sign of break through.

Ensure the carabiner gate opens, closes and locks properly.

Check the springs on the top plate spring loaded wheels.  Ensure the springs move the

wheels in toward the rail.

Ensure all wheels (friction wheel and bearing) rotate freely.

Ensure the orientation lock is functioning and free from debris.

Ensure all teeth or clamping surfaces do not have excessive signs of wear.

Ensure that the carabiner has not been deployed and is in one piece.

Comments:______________________________________________________________________________

Illustration on back of page

SAF-T-GRAB ILLUSTRATION