4010

Serious Injury

4/14/04

Edina

Baker Bakery

2051

311812

30

Employee walking between bun and bread

machines when he slipped and fell, his arm

entered the uncovered area on the side of the

bread machine that had an exposed chain and

sprocket. The bread machine was operating

and entangled the victim’s forearm in the

machine.

Serious citation issued for failure to enclose

sprocket wheels or chains seven feet or less

above floors or platforms [1910.219(f)(3)]and

failure to maintain injury and illness logs

[1904.29(a)].

4009

Serious Injury

4/8/04

Eagan

Maintenance Mtg. Wood Products

2439

321214

190

Employee suffered severe burns while

completing the electrical installation of a

saw. He was making electrical connections

inside the switch enclosure (switch was

energized) when his uninsulated wrench or

pliers slipped and made contact between

adjacent conductors, or conductors and

ground.

Serious citations for: inadequate LOTO

procedures [1910.147(c)(4)(i) & (ii)], lack of

LOTO training [1910.147(c)(7)(i)]; lack of

ESWP training [1910.333(b)(1)], lack of ESWP

procedures [1910.333(b)(2)(i)], unqualified

persons allowed to work on energized electric

equipment [1910.333(c)(2)]; lack of proper

PPE, PPE not maintained, insulated hand tools

not used [1910.335(a)(1)(i), (ii), & (v), and

(a)(2)(i)