Pre-Job Planning and Safety Analysis Outline

- Review the work to be done, considering both productivity and safety:

a. Break the job down into manageable tasks.

b. How is each task to be done?

c. In what order are tasks to be done?

d. What equipment or materials are needed?

e. Are any particular skills required?

- Clearly assign responsibilities.

- Who is to perform the pre-use of equipment tests?

- Will the work require: a hot work permit; a confined entry permit, lockout/tagout (of what equipment or machinery)?

- Will it be necessary to barricade for clear work zones?

- Will aerial lifts be required?

- What personal protective equipment will be needed?

- Will fall protection be required?

- What are the hazards in each task? Consider –

Access		Work at heights	Work at depths	Fall Hazards

Worker position	Worker posture	Twisting, bending	Weight of objects

Elevated loads	Welding		Fire			Explosion

Electricity		Chemicals		Dusts			Noise

Weather           	Sharp objects	Steam			Vibration

Stored energy	Dropping tools	Pressure		Hot objects

Forklift trucks	Conveyors		Moving equipment	Machine guarding

10.  Of the hazards identified, do any present severe risk of injury?

11.  Develop hazard control measures, applying the Safety Decision Hierarchy.

- Eliminate hazards and risks through system and work methods design and redesign

- Reduce risks by substituting less hazardous methods or materials

- Incorporate safety devices (fixed guards, interlocks)

- Provide warning systems

- Apply administrative controls (work methods, training, etc.)

- Provide personal protective equipment

12.  Is any special contingency planning necessary (people, procedures)?

13.  What communication devices will be needed (two-way, hand signals)?

14.  Review and test the communication system to notify the emergency team (phone number, responsibilities).

15.  What are the workers to do if the work doesn’t go as planned?

16.  Considering all of the foregoing, are the risks acceptable? If not, what action should be taken?

Upon Job Completion

17. Account for all personnel

18. Replace guards

19. Remove safety locks

20. Restore energy as appropriate

21. Remove barriers/devices to secure area

22. Account for tools

23. Turn in permits

24. Clean the area

25. Communicate to others affected that the job is done

26. Document all modifications to prints and appropriate files