TURN OVER TEST CONTINUES ON BACK

HEARING CONSERVATION

PRETEST

NAME:_____________________________DATE:________________SCORE:____/12

(Print)

- “Ringing of the ears” is also known as what?

- Tinnitus

- Conductive Hearing Loss

- Sensorineural Hearing Loss

- Permanent Threshold Shift

- What does the PEL for noise exposure measure in units?

  - Sound Pressure Level

  - Frequency

  - Sound Intensity

  - Amplitude

- Baseline audiograms must be conducted ______________.

- Prior to being exposed to on the job noise over the action level

- Within one month of being exposed to on the job noise over the action level

- Prior to being exposed to on the job noise over the Permissible Exposure Level

- Within one month of being exposed to on the job noise over the Permissible Exposure Level

- None of the above

- Limiting noise exposure duration is an example of which kind of control?

- Elimination or Substitution

- Engineering

- Administrative

- PPE

- Which would be the most effective method to control occupational noise exposures?

- Installing noise attenuating curtains

- Enclosing the source

- Increasing worker distance from the source

- Training employees on protective measures.

- OSHA requires a hearing conservation program at what level of occupational noise exposure?

- 100 dBA, 8hr TWA

- 90 dBA, 8hr TWA

- 85 dBA, 8hr TWA

- 80 dBA, 8hr TWA

- Under OSHA’s Hearing conservation standard, how many hours per day are you permitted to be exposed to a sound level of 90dBA?

  - 0.25 hours or less

  - 1 hour

  - 4 hours

  - 8 hours

- (T/F)  Foam earplugs will not provide the field attenuation in dB printed on the container as the Noise Reduction Rating (NRR.)

- (T/F)  The use of ear plugs in combination with ear muffs can provide greater protection than using either by itself.

- This frequency of noise is more difficult to shield or attenuate

  - High

  - Low

  - Mid

- This frequency of noise is more likely to cause hearing damage

  - High

  - Low

  - Mid

- What rating is used to measure the estimated attenuation capacity of a hearing protector?

- Maximum Exposure Rating

- Protection Limit Rating

- Noise Reduction Rating