Tip of the Week

December 13, 2004

How Long, How Much and How Hot?

True or False?  Method 5 specifies that a particulate test must be done for at least 60 minutes, must collect 30 dscf and must have the filter heated to 248±25°F.

Well, not exactly.

Method 5 says nothing about how long a test should be, except that it should equal or exceed the minimum time specified in the regulations, and be long enough to collect the minimum sample volume, also specified in the regulations.  As for the filter temperature, it says that the temperature around the filter holder must be kept at 248±25°F, or some other temperature…once again, as specified in the regulations.  It goes on to say that you can operate at any temperature lower than that specified (the theory here is that lower temperatures could only bias the results higher, which is OK from the EPA’s point of view).

So what are these other regulations that contain all of this crucial information about how to run the test?  One source is the New Source Performance Standards (NSPS), which comprise over half of the bulk of your Part 60 CFR.  These are the “subparts”, as they are generally called, and are specific for each different type of industry.  An on-line version of the NSPS is available at http://www.gpoaccess.gov/ecfr/index.html.

Usually, each subpart contains a section called “Test Methods and Procedures”, which will provide the required method and pertinent testing specifications for any pollutant that is limited by the regulation.  The attached table contains a handy summary of information pertaining to particulate testing at most major industry classifications regulated under Part 60.

Other potential regulations that influence the way the test is performed include the National Emissions Standards for Hazardous Air Pollutants (NESHAPS), as well as specific state or local regulations.  Normally, the facility’s operating permit will specify the applicable regulations that it must follow.